Design and Performance Evaluation of a Secure Smart Hospital Network for Telemedicine and Critical Care

1. Problem Understanding and Formulation

1.1 Problem Overview

The proposed system is a modern computer network for a multi-specialty hospital consisting of three interconnected buildings:

- Emergency & ICU Block

- Diagnostic & Imaging Block

- Administrative & OPD Block

The network must support critical hospital applications such as real-time telemedicine, continuous monitoring of IoT-enabled medical devices, transfer of large DICOM medical images, electronic medical records, administrative services, and emergency alerts.

The existing network has several problems, including high packet loss in wireless areas near MRI rooms, slow routing updates during network/device failures, TCP retransmission storms during peak telemedicine usage, DNS lookup delays, insecure file transfers, and unreliable email alerts.

Therefore, the objective is to design a reliable, low-latency, secure, scalable and fault-tolerant hospital network using concepts from the Data Link, Network, Transport and Application layers.

1.2 Network Entities

A. Buildings

| Building | Major Departments | Main Network Requirements |
| --- | --- | --- |
| Emergency & ICU Block | ICU, Emergency, Patient Monitoring | Very low latency, high reliability, highest priority |
| Diagnostic & Imaging Block | Radiology, MRI, CT, Imaging | High bandwidth and reliable DICOM transfer |
| Administrative & OPD Block | OPD, Reception, Billing, Administration | Reliable general-purpose and administrative traffic |

B. Device Types

The network contains several types of devices:

- Patient monitors

- Infusion pumps

- Wearable medical sensors

- MRI and CT systems

- Radiology workstations

- Doctor/nurse computers

- IP phones

- Telemedicine systems

- Wireless access points

- Servers

- Network switches and routers

- Printers

- Administrative PCs

- Guest devices

Different devices generate different traffic patterns. For example, patient monitors generate small but frequent data packets, while MRI systems generate very large DICOM files.

1.3 Traffic Classes

Hospital traffic is divided into classes according to its importance.

| Priority | Traffic | Example | Requirement |
| --- | --- | --- | --- |
| 1 – Highest | Emergency/ICU | Patient monitoring, emergency alerts | Very low latency, high reliability |
| 2 | Telemedicine | Real-time video consultation | Low latency and low jitter |
| 3 | Medical IoT | Sensors, infusion pumps | Reliable continuous communication |
| 4 | DICOM | MRI/CT image transfer | High throughput and reliability |
| 5 | Administration | Billing, records, email | Normal reliability |
| 6 – Lowest | Guest | Guest Internet access | Best-effort |

Priority-based QoS is therefore required so that emergency and telemedicine traffic is not delayed by large imaging transfers or guest traffic.

1.4 Failure Modes

The important failure modes in the existing hospital network are:

1. Wireless packet loss

MRI and other imaging equipment can create a difficult RF environment. Wireless clients near these areas may experience interference and packet loss.

Solution: Wi-Fi 6/802.11ax, appropriate AP placement, channel planning, OFDMA and QoS.

2. Link failure

A fibre or network link between buildings may fail.

Solution: redundant fibre links and dynamic routing using OSPF.

3. Router/switch failure

A routing device may become unavailable.

Solution: redundant network devices and dynamic routing.

4. Network congestion

During peak telemedicine hours, excessive traffic can cause queue buildup and TCP retransmissions.

Solution: QoS, priority queuing, traffic classification and congestion-control analysis.

5. DNS failure/delay

Slow or unavailable DNS services can delay application access.

Solution: hierarchical DNS, caching and redundant DNS servers.

6. File-transfer security failure

Plain FTP can expose sensitive medical information.

Solution: secure transfer using HTTPS/TLS, SFTP or DICOM over TLS.

7. Email alert failure

Critical medical alerts may fail to reach authorized staff.

Solution: authenticated SMTP over TLS, retries, logging and backup notification mechanisms.

8. Unauthorized access

Guest or compromised IoT devices could attempt to access ICU or administrative systems.

Solution: VLAN segmentation, ACLs, firewall policies, authentication and least-privilege access.

1.5 Assumptions

The following assumptions are made for the network design:

- Each building has multiple departments and network users.

- Fibre is available for the inter-building backbone.

- Ethernet is used for fixed high-performance devices.

- Wi-Fi 6 is used for mobile/ward devices.

- BLE is used for suitable wearable medical sensors.

- IPv4 is used as the primary addressing scheme; IPv6 can be added later.

- OSPF is used as the preferred dynamic routing protocol.

- Medical IoT devices are placed in a dedicated VLAN.

- Critical hospital traffic receives higher QoS priority.

- Servers are placed in a separate server VLAN.

- Guest Wi-Fi is isolated from hospital networks.

- Security policies follow HIPAA-like principles for confidentiality, integrity and availability.

- The simulation uses representative traffic rather than actual patient information.

- Network performance is evaluated using latency, packet loss, throughput, jitter and retransmissions.

1.6 Constraints

The design must consider the following constraints:

- Patient-care traffic must have high availability.

- Emergency traffic must experience minimal delay.

- Large DICOM files require high bandwidth.

- Wireless coverage must be reliable in wards.

- Medical data must be protected from unauthorized access.

- The network should remain operational during individual link/device failures.

- The design should be scalable when new departments or devices are added.

- The solution should control implementation and operational costs.

- Battery-powered medical sensors should use energy-efficient communication where possible.

- The simulation cannot perfectly reproduce all real-world hospital RF interference and medical-device behaviour.

2. Application of Course Knowledge

The hospital network applies concepts from all four networking layers to improve reliability, performance and security.

Data Link Layer

- CRC: Used to detect transmission errors in medical/network data.

- Hamming Code: Demonstrates error correction for single-bit errors.

- Wi-Fi 6/OFDMA: Reduces wireless congestion and improves communication for multiple medical devices.

- Ethernet/Fibre: Provides reliable high-speed wired and inter-building connectivity.

Network Layer

- IPv4 Subnetting: Separate networks are created for ICU, Radiology, OPD, Administration and Medical IoT.

- Example: 10.10.10.0/24 provides 254 usable addresses.

- OSPF: Selected instead of Distance Vector routing because it provides faster convergence after link failures.

- ICMP: Used with ping/traceroute to test connectivity and diagnose faults.

Transport Layer

- TCP: Used for reliable medical records, DICOM transfers and administrative data.

- UDP: Used for latency-sensitive telemetry and real-time communication.

- QUIC: Provides secure, reliable and low-latency communication for modern applications.

- QoS: ICU and emergency traffic receives the highest priority.

Queuing Analysis

For an M/M/1 queue:

- Arrival rate: λ = 80 packets/s

- Service rate: μ = 100 packets/s

Utilization:

ρ = λ/μ = 80/100 = 0.8 = 80%

Average delay:

W = 1/(μ − λ) = 1/(100 − 80) = 0.05 s = 50 ms

This shows that as traffic approaches the link's service capacity, queueing delay increases, demonstrating the importance of QoS and congestion control.

Application & Security

- DNS: Hierarchical DNS and caching reduce lookup delays.

- HTTPS/TLS 1.3: Protects telemedicine and patient information.

- Secure DICOM: Protects medical imaging during transfer.

- SMTP/TLS: Provides secure emergency email alerts.

- SNMPv3: Monitors network devices and infrastructure.

- VLANs + ACLs: Isolate ICU, IoT, Admin and Guest traffic.

Overall, these networking concepts provide low latency, reliability, security, fault tolerance and scalability for the hospital network.

Solution/design/methodology;

Student A — Lead / Topology

Based on the team-role allocation, Student A is responsible for the overall network design, building the complete topology in Packet Tracer, the addressing plan, VLAN configuration, and wireless AP deployment. The hospital assignment specifically requires a hierarchical, hybrid topology spanning three buildings, department-wise VLAN segmentation, and a modern WLAN standard (Wi-Fi 6 / 802.11ax) for ward and diagnostic-area connectivity.

## 1. Network Topology Design

The hospital network is designed as a three-tier hierarchical (Core – Distribution – Access) topology, which is itself a hybrid of star and extended-star/tree topologies. Each of the three buildings — Emergency & ICU Block, Diagnostic & Imaging Block, and Administrative & OPD Block — is modelled as its own distribution zone, so that a failure or upgrade in one building does not disrupt the others. Within each building, access-layer switches and wireless access points connect end devices in a star pattern, and the buildings are tied together through a single collapsed core switch.

- Core Layer: CORE-SW (3560-24PS Multilayer Switch) — single high-speed backbone point interconnecting all three buildings; carries inter-building and inter-VLAN traffic.

- Distribution Layer: ICU-DIST, DIAG-DIST, ADMIN-DIST (3560-24PS Multilayer Switches) — one per building, aggregates access-layer traffic, performs inter-VLAN routing, and enforces department-level policy.

- Access Layer: ICU-ACCESS, DIAG-ACCESS, ADMIN-ACCESS (2960-24TT Switches) — connects wired end devices and access points within each building.

- Wireless Layer: 6 Access Points (2 per building) — provides Wi-Fi coverage to ward staff, laptops and mobile devices in each block.

- End Devices: PC-ICU-01–03, PC-RAD-01–03, PC-OPD-01–02, PC-ADMIN-01, Laptop0–5 — ward/ICU PCs, radiology PCs, OPD/admin PCs and staff laptops.

- Servers: DNS Server, Web Server, Email Server, DIACOM (DICOM imaging) Server — centralised application services placed in the Admin block server farm.

This design keeps the three physical/functional buildings separate at the distribution layer while still allowing every device in the hospital to reach shared services (DNS, web, email, DICOM) through the core, satisfying the requirement for a scalable, hybrid, multi-building topology. The full topology, including all switches, access points, end devices and servers, was built and tested end-to-end in Cisco Packet Tracer.

Figure 1: Three-building hierarchical hospital network topology (Cisco Packet Tracer)

## 2. VLAN Design (per Department)

VLANs are configured on the access and distribution switches to logically separate departmental traffic, reduce broadcast domains, and align with the addressing plan used for subnetting. The VLAN numbering below maps directly onto Student B's IP subnetting plan (10.10.0.0/16) so that each VLAN corresponds one-to-one with a department subnet.

- VLAN 10 — ICU & Emergency (ICU-DIST) — mapped to 10.10.0.0/22

- VLAN 20 — Diagnostic & Imaging / Radiology (DIAG-DIST) — mapped to 10.10.4.0/22

- VLAN 30 — OPD Network (ADMIN-DIST) — mapped to 10.10.8.0/22

- VLAN 40 — Administration (ADMIN-DIST) — mapped to 10.10.12.0/23

- VLAN 50 — IoT Medical Devices, monitors, pumps, wearables (ICU-DIST / DIAG-DIST) — mapped to 10.10.14.0/22

- VLAN 60 — Servers: DNS, Web, Email, DIACOM (ADMIN-DIST) — mapped to 10.10.18.0/24

- VLAN 99 — Network Management / native VLAN (CORE-SW) — mapped to 10.10.19.0/24

Access ports on ICU-ACCESS, DIAG-ACCESS and ADMIN-ACCESS are assigned to the relevant VLAN using:

Switch(config)# interface range fa0/1-8

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

Trunk links between each access switch and its distribution switch, and between each distribution switch and CORE-SW, are configured with 802.1Q trunking so that all department VLANs can reach the core for inter-VLAN routing and access to the server farm.

## 3. Wireless LAN Design (Wi-Fi 6 / 802.11ax)

Six access points are deployed, two per building, to provide ward-wide and department-wide wireless coverage for staff laptops, tablets and IoT medical devices while keeping wired PCs on the access switches. Wi-Fi 6 (802.11ax) is chosen over legacy 802.11n/ac because it uses OFDMA to serve many simultaneous low-data-rate IoT sensors efficiently, supports Target Wake Time (TWT) to extend the battery life of wearable medical devices, and still relies on CSMA/CA for contention-based medium access, which the assignment requires to be justified.

- Emergency & ICU Block: AccessPoint0, AccessPoint1 — SSID HOSP-ICU-WLAN — mapped to VLAN 10 / 50.

- Diagnostic & Imaging Block: AccessPoint2, AccessPoint3 — SSID HOSP-DIAG-WLAN — mapped to VLAN 20 / 50.

- Administrative & OPD Block: AccessPoint4, AccessPoint5 — SSID HOSP-ADMIN-WLAN — mapped to VLAN 30 / 40.

Placing two APs per block (rather than one) also reduces the packet loss seen near MRI/imaging rooms mentioned in the problem statement, by giving client devices a nearer AP to associate with and avoid RF interference from imaging equipment.

## 4. Alternative Topologies Considered

Two alternative designs were evaluated before finalising the hierarchical hybrid topology:

- Flat single-switch star topology — all devices connected to one central switch. Rejected because a single point of failure would take down all three buildings at once, and it does not scale to 2,000+ wireless clients and 300 IoT sensors.

- Full-mesh topology between three building switches — every building switch directly connected to every other. Rejected because it adds unnecessary cabling/fibre cost between buildings for only three nodes, and does not provide the clean Core–Distribution–Access separation needed for VLAN-based departmental policy and SDN/policy-based traffic prioritisation.

Therefore, the three-tier hierarchical/hybrid topology is preferred because it isolates departments through VLANs, keeps each building's failures local, supports the 802.1Q trunking and OSPF routing configured by Student B, and gives a single, clear point (the core) at which SDN/policy-based prioritisation of critical ICU traffic can be applied.

## 5. Student A Contribution Summary

Student A contributed to the overall network design by building the complete three-building hierarchical topology in Cisco Packet Tracer (core, distribution and access switches, six Wi-Fi 6 access points, all end devices and the shared server farm), defining the department-wise VLAN scheme aligned with the team's IP addressing plan, configuring access and trunk ports, and deploying the wireless access points per department. This topology forms the physical and logical foundation on which Student B's routing/OSPF configuration, Student C's Wireshark capture points, and Student D's application services were implemented and tested.

This directly matches the assigned responsibility of Student A: overall network design, Packet Tracer topology, addressing plan, VLANs, and wireless APs.

## Student B – Routing & Network Layer

Based on the team-role allocation, Student B is responsible for IP subnetting, routing configuration and testing, SDN concepts, and ICMP diagnostics. The hospital assignment specifically requires hierarchical addressing, subnetting for departments, routing comparison, ICMP diagnostics, and policy-based prioritisation.

### 1. IP Addressing and Subnetting Plan

A hierarchical IPv4 addressing scheme can be used with the private network block 10.10.0.0/16.

| Department / Network | Network Address | Subnet Mask | Usable Hosts |
| --- | --- | --- | --- |
| ICU & Emergency | 10.10.0.0/22 | 255.255.252.0 | 1022 |
| Diagnostic & Imaging | 10.10.4.0/22 | 255.255.252.0 | 1022 |
| OPD Network | 10.10.8.0/22 | 255.255.252.0 | 1022 |
| Administration | 10.10.12.0/23 | 255.255.254.0 | 510 |
| IoT Medical Devices | 10.10.14.0/22 | 255.255.252.0 | 1022 |
| Servers | 10.10.18.0/24 | 255.255.255.0 | 254 |
| Network Management | 10.10.19.0/24 | 255.255.255.0 | 254 |

This addressing scheme separates hospital departments and improves network scalability, security, and traffic management. VLANs can be associated with each department to reduce unnecessary broadcast traffic.

### 2. Routing Protocol Selection

For the Smart Hospital Network, OSPF (Open Shortest Path First) is selected as the primary routing protocol.

#### Why OSPF?

- It is a Link-State routing protocol.

- Provides fast routing convergence during network failures.

- Suitable for large and scalable networks.

- Supports hierarchical network design using areas.

- Uses cost as its routing metric.

- Helps achieve the requirement of rapid recovery during equipment or link failures.

### Distance Vector vs Link State Routing

Feature; Distance Vector; Link State (OSPF).

Network Knowledge; Limited; Complete topology view.

Convergence; Slower; Faster.

Updates; Periodic; Triggered updates.

Scalability; Lower; High.

Example; RIP; OSPF.

Hospital Suitability; Less suitable; Highly suitable.

Therefore, OSPF is preferred because the hospital requires routing convergence in less than 3 seconds during a single link failure.

### 3. Basic OSPF Configuration

Example router configuration:

Router> enable

Router# configure terminal

Router(config)# router ospf 1

Router(config-router)# network 10.10.0.0 0.0.255.255 area 0

Router(config-router)# end

Each router in the three-building hospital topology can be configured with OSPF Area 0 for communication between:

- Emergency & ICU Block

- Diagnostic & Imaging Block

- Administrative & OPD Block

The routing tables can be verified using:

Router# show ip route

Router# show ip ospf neighbour

Router# show ip protocols

### 4. Routing Convergence Test

To test routing reliability:

- Establish communication between devices in different buildings.

- Use the ping command to verify connectivity.

- Disconnect one inter-router link.

- Observe OSPF selecting an alternate path.

- Measure the time required for routing convergence.

- Restore the link and verify that the routing table updates correctly.

The expected result is that the network should automatically reroute traffic through an alternative path with minimal interruption.

### 5. ICMP Diagnostics

ICMP is used for network troubleshooting and connectivity verification.

Important commands include:

PC> ping 10.10.18.10

PC> tracert 10.10.18.10

Ping verifies whether the destination device is reachable, while Traceroute identifies the path taken by packets through different routers.

ICMP diagnostics help identify:

- Unreachable devices

- Routing failures

- Link failures

- Excessive delay

- Incorrect IP configuration

### 6. SDN / Policy-Based Prioritisation Concept

A simple Software Defined Networking (SDN) or policy-based approach can be introduced to prioritise critical hospital traffic.

Traffic priority can be arranged as:

- Emergency ICU monitoring data – Highest Priority

- Telemedicine video traffic – High Priority

- Medical imaging transfer – Medium Priority

- Administrative traffic – Normal Priority

The SDN controller or policy-based routing mechanism can dynamically manage traffic based on its importance. This ensures that emergency patient-monitoring traffic receives faster delivery even during periods of high network congestion.

### 7. Student B Contribution Summary

Student B contributed to the Network Layer design by creating the IP subnetting table, configuring OSPF routing, testing routing convergence during link failures, performing ICMP-based diagnostics, and proposing an SDN/policy-based traffic prioritisation mechanism for critical-care communication. The routing design ensures scalable communication between the three hospital buildings while providing fast recovery and reliable connectivity for emergency medical services.

This directly matches the assigned responsibility of Student B: subnetting tables, routing configuration and convergence tests, and an SDN concept note.

Student C — Transport Layer & Performance Analysis

1. Introduction

The transport layer is responsible for end-to-end communication between hospital devices, servers, and users. The hospital network carries different types of traffic, including real-time telemedicine video, patient-monitoring data, DICOM medical images, medical records, DNS requests, and administrative services.

These applications have different requirements. For example, telemedicine requires low latency and low jitter, whereas medical records and DICOM files require reliable and complete delivery.

Therefore, this project evaluates TCP, UDP, and QUIC, together with congestion control and Quality of Service (QoS), to determine the most suitable transport approach for each hospital application.

2. TCP

Transmission Control Protocol (TCP) is a connection-oriented and reliable transport protocol.

TCP provides:

- Reliable delivery

- Ordered delivery

- Error detection

- Retransmission of lost segments

- Flow control

- Congestion control

TCP communication

Client                         Server

|                              |

|-------- SYN ---------------->|

|<------ SYN + ACK ------------|

|-------- ACK ---------------->|

|                              |

|====== Data transfer =========>|

|<========= ACK ===============|

This three-way handshake establishes a TCP connection.

Hospital applications suitable for TCP

TCP is appropriate for:

- Electronic medical records

- DICOM file transfer

- Database communication

- Web applications

- Email

- Administrative applications

For these applications, losing or reordering data could result in an incomplete or corrupted transaction.

3. TCP Congestion Control

TCP attempts to prevent network congestion by adjusting its sending rate.

A simplified TCP congestion-control process is:

Increase sending rate

↓

Network becomes congested

↓

Packet loss / duplicate ACK

↓

TCP reduces sending rate

↓

Retransmission

↓

Gradual increase again

During peak telemedicine traffic, many simultaneous TCP flows can compete for bandwidth.

For example:

DICOM transfer

|

|

Telemedicine +----> Core Router ----> Server

|

|

Medical records

If the network becomes congested:

High traffic

↓

Queue fills

↓

Packets dropped

↓

TCP retransmissions

↓

Additional traffic

↓

More congestion

This can contribute to the TCP retransmission storms observed in the existing hospital network.

4. UDP

User Datagram Protocol (UDP) is connectionless and has very low protocol overhead.

Unlike TCP, UDP does not inherently provide:

- Guaranteed delivery

- Packet ordering

- Retransmission

- Congestion control equivalent to TCP

UDP communication

Sender                         Receiver

|                              |

|-------- UDP packet --------->|

|-------- UDP packet --------->|

|-------- UDP packet --------->|

There is no TCP-style three-way handshake.

Hospital applications

UDP is suitable for traffic where timeliness is more important than perfect delivery, such as:

- Real-time telemetry

- Voice/video components

- Certain real-time monitoring applications

For example, if one video packet is lost:

Video packets:

1  2  3  X  5  6

↑

lost

Waiting for retransmission may be worse for a real-time video stream than continuing with a small amount of loss.

5. QUIC

QUIC is a modern transport protocol built over UDP.

It provides features including:

- Reliable delivery

- Encryption

- Multiplexed streams

- Reduced connection-establishment overhead

- Modern congestion control

QUIC is particularly relevant to modern web and telemedicine applications.

A simplified architecture is:

Telemedicine Application

↓

QUIC

↓

UDP

↓

IP Network

Compared with traditional TCP + TLS, QUIC integrates transport security more closely into the protocol architecture.

Possible hospital use

QUIC can be evaluated for:

- Telemedicine portals

- Real-time web applications

- Modern HTTPS/HTTP/3 services

- Remote consultation platforms

6. TCP vs UDP vs QUIC

| Feature | TCP | UDP | QUIC |
| --- | --- | --- | --- |
| Connection-oriented | Yes | No | Yes |
| Reliable delivery | Yes | No | Yes |
| Ordered streams | Yes | No | Yes |
| Retransmission | Yes | No | Yes |
| Congestion control | Yes | Not inherently | Yes |
| Encryption | Usually TLS above TCP | Not inherent | Built into QUIC |
| Connection setup | Higher | Very low | Low |
| Best use | Files/records | Real-time traffic | Modern real-time/web applications |
| Hospital example | DICOM | Telemetry | Telemedicine portal |

Conclusion

There is no single protocol that is best for every hospital application.

TCP → reliable data

UDP → time-sensitive traffic

QUIC → modern encrypted low-latency application

7. Quality of Service (QoS)

The hospital network must give priority to critical medical traffic.

Without QoS, traffic may compete equally:

ICU monitoring ──────┐

Telemedicine ────────┤

DICOM ────────────────┼──> Router → Congested link

Administration ──────┤

Guest downloads ─────┘

With QoS:

Router

|

+--------+--------+

|                 |

High priority      Low priority

|                 |

ICU/telemedicine    Admin/Guest

medical telemetry   bulk traffic

Proposed priority

| Priority | Traffic | Reason |
| --- | --- | --- |
| 1 | ICU/Emergency | Patient safety |
| 2 | Telemedicine | Real-time communication |
| 3 | Medical IoT | Continuous monitoring |
| 4 | DICOM | Important but delay-tolerant |
| 5 | Administration | Normal business traffic |
| 6 | Guest Wi-Fi | Lowest priority |

8. Priority Queuing

Priority queuing allows packets from critical applications to be transmitted before lower-priority packets.

Example:

Incoming packets

↓

+---------------------+

| Priority Class 1    | ← ICU

+---------------------+

| Priority Class 2    | ← Telemedicine

+---------------------+

| Priority Class 3    | ← Medical IoT

+---------------------+

| Priority Class 4    | ← DICOM

+---------------------+

| Priority Class 5    | ← Admin

+---------------------+

| Priority Class 6    | ← Guest

+---------------------+

↓

Outgoing link

During congestion, lower-priority traffic may experience greater delay, but critical traffic continues to receive network resources.

9. Performance Metrics

Student C should evaluate the network using the following metrics.

9.1 Latency

Latency is the time required for a packet to travel from source to destination.

Latency=Tarrival-Tdeparture

Lower latency is particularly important for:

- telemedicine

- ICU monitoring

- emergency communication

9.2 Packet Loss

Packet loss is:

Packet Loss%=Packets Sent-Packets ReceivedPackets Sent×100

Example:

If 10,000 packets are sent and 50 are lost:

Packet Loss=5010000×100=0.5%

9.3 Throughput

Throughput represents the amount of useful data successfully transmitted per unit of time.

Throughput=Data transferredTime

It is particularly important for DICOM imaging because medical images can be large.

9.4 Jitter

Jitter is the variation in packet arrival time.

For example:

Packet       Arrival interval

1            10 ms

2            11 ms

3            10 ms

4            25 ms

5            12 ms

The sudden 25 ms interval produces increased jitter.

Low jitter is important for real-time video and voice.

9.5 TCP Retransmissions

Count the number of TCP segments that are retransmitted.

High retransmissions indicate:

- packet loss

- congestion

- poor link quality

- insufficient network capacity

The objective of QoS is to prevent critical traffic from suffering excessive congestion and loss.

10. Wireshark Analysis

Wireshark can be used to capture and inspect network packets during the simulation/emulation.

Useful display filters include:

tcp

udp

quic

tcp.analysis.retransmission

tcp.analysis.duplicate_ack

dns

icmp

TCP analysis

In Wireshark, identify:

- SYN

- SYN-ACK

- ACK

- Data segments

- ACKs

- Retransmissions

- Duplicate ACKs

A typical TCP sequence can be represented as:

SYN

↓

SYN-ACK

↓

ACK

↓

DATA

↓

ACK

↓

DATA

↓

ACK

If packets are lost:

DATA 100

DATA 101

DATA 102  ← lost

DATA 103

↓

Duplicate ACK

↓

Retransmission of DATA 102

This provides direct evidence of congestion or packet loss.

11. Simulation Experiment 1 — TCP Congestion

Objective

Determine how TCP behaves when the network is heavily loaded.

Procedure

Generate:

- multiple TCP flows

- DICOM transfers

- administrative traffic

- telemedicine traffic

Then observe:

- packet loss

- retransmissions

- throughput

- delay

Expected observation

As network utilisation increases:

Traffic↑⇒Queueing↑⇒Packet loss↑⇒TCP retransmissions↑

This demonstrates why uncontrolled bulk traffic can negatively affect hospital applications.

12. Simulation Experiment 2 — QoS OFF vs QoS ON

This should be one of your main experiments.

Case A — QoS disabled

DICOM ───────────┐

Telemedicine ────┤

ICU IoT ─────────┼──> Same queue

Admin ───────────┤

Guest ───────────┘

Measure:

- ICU latency

- telemedicine jitter

- packet loss

- DICOM throughput

- TCP retransmissions

Case B — QoS enabled

ICU ───────────────→ HIGH priority

Telemedicine ──────→ HIGH priority

IoT ───────────────→ HIGH priority

DICOM ─────────────→ MEDIUM

Admin ─────────────→ LOW

Guest ─────────────→ LOW

Run exactly the same traffic again.

Then compare the results.

14. Simulation Experiment 3 — TCP vs UDP

Generate equivalent traffic using TCP and UDP.

TCP

Observe:

- reliable delivery

- acknowledgements

- retransmissions

- congestion control

UDP

Observe:

- lower overhead

- no handshake

- no inherent retransmission

- packet loss when congestion occurs

Analysis

TCP may maintain reliability at the cost of additional delay when packets are lost.

UDP can maintain low overhead and latency but may sacrifice reliability.

Therefore:

For real-time patient telemetry, controlled UDP-based communication can be advantageous when the application can tolerate limited packet loss or implements its own recovery mechanism. For medical records and DICOM, reliable transport is preferable.

15. Simulation Experiment 4 — Routing Failure and Transport Behaviour

Although routing is primarily Student B's responsibility, Student C should analyse its transport-layer impact.

Normal:

ICU → Core 1 → Diagnostic Server

Failure:

ICU → Core 1 X

↓

ICU → Core 2 → Diagnostic Server

Observe in Wireshark:

- packet loss

- TCP retransmission

- temporary delay

- connection recovery

- UDP packet loss

This connects the Network Layer work with Student C's transport analysis.

16. Queuing Analysis

A simple M/M/1 model can be used to explain congestion.

Let:

λ=packet arrival rate
μ=packet service rate

The utilisation is:

ρ=λμ

For stable operation:

ρ<1

The average time in the system is:

W=1μ-λ

For example, assume:

λ=700 packets/s

and:

μ=1000 packets/s

Then:

ρ=7001000=0.7

and:

W=11000-700=0.00333s

Therefore:

W≈3.33ms

If traffic increases closer to the service capacity, queueing delay increases rapidly.

This explains why the hospital experiences significant delays during peak telemedicine periods.

17. Final Analysis

The transport-layer analysis demonstrates that different hospital applications require different communication strategies.

TCP provides reliable and ordered delivery and is appropriate for DICOM, medical records, databases, and administrative services. However, heavy congestion can result in retransmissions and increased delay.

UDP has lower overhead and is appropriate for latency-sensitive telemetry and real-time media when the application can tolerate some loss or provides its own recovery mechanism.

QUIC provides reliable, encrypted transport over UDP and is a strong option for modern telemedicine web applications where reduced connection overhead and multiplexing are beneficial.

QoS is essential because not all hospital traffic has equal importance. ICU monitoring, emergency traffic, and telemedicine should receive higher priority than administrative and guest traffic.

18. Student C Conclusion

Student C Conclusion:
The transport-layer evaluation shows that the hospital network should not use a single transport protocol for all applications. TCP is appropriate for reliable medical records and DICOM transfers, while UDP is suitable for latency-sensitive telemetry and real-time media where occasional packet loss can be tolerated. QUIC provides a modern alternative for secure, low-latency web-based telemedicine applications. Wireshark analysis can be used to identify TCP retransmissions, duplicate acknowledgements, packet loss, DNS delays, and transport behaviour during congestion. The QoS experiment demonstrates that prioritising ICU, emergency, telemedicine, and medical IoT traffic can protect critical services during periods of heavy network utilisation. Therefore, combining appropriate transport protocols with priority queuing provides better latency, reliability, and overall performance for the hospital's critical applications.

Student D — Application Layer, Security & Broader Impact

For your hospital network project, Student D should handle the Application Layer + Security + Privacy + Social/Sustainability impact.

Your work can be divided into:

- DNS

- HTTPS/TLS 1.3

- Secure DICOM/file transfer

- SMTP email alerts

- SNMP monitoring

- Security and privacy

- Energy efficiency

- Patient safety

- Cost and sustainability

- SDG 3, 9 and 11

1. Application Layer Architecture

The hospital should have a dedicated Server VLAN containing the main application services.

SERVER VLAN

|

+----------+-----------+-----------+----------+

|          |           |           |          |

DNS       Web        PACS        SMTP       SNMP

Server     Server      Server      Server    Monitor

|          |           |           |

|       Telemedicine DICOM      Alerts

|         Portal      Files

|          |

+----------+----------------------+

This separation prevents ordinary user traffic from directly accessing critical servers.

2. DNS Design

DNS converts names into IP addresses.

For example:

telemedicine.hospital.local

↓

10.10.70.20

Proposed DNS hierarchy

hospital.local

│

├── dns.hospital.local

├── telemedicine.hospital.local

├── pacs.hospital.local

├── mail.hospital.local

├── monitoring.hospital.local

├── icu.hospital.local

├── radiology.hospital.local

└── opd.hospital.local

Why DNS is important

The existing network has DNS lookup delays, which can slow down application access.

Use:

- local DNS caching

- redundant DNS servers

- appropriate DNS records

- controlled external DNS forwarding

For example:

Client

|

| DNS query

↓

Local DNS resolver

|

+---- Cached? ---- Yes → immediate response

|

No

↓

Authoritative/external DNS

DNS experiment

Compare:

Test 1: First/uncached lookup

Test 2: Repeated/cached lookup

Record:

| Test | Lookup time |
| --- | --- |
| First DNS lookup | ___ ms |
| Cached lookup | ___ ms |

Report statement

A hierarchical and caching DNS architecture is proposed to reduce name-resolution latency and improve application responsiveness. Redundant DNS infrastructure can also improve availability during server failures.

3. HTTPS and TLS 1.3

The telemedicine portal contains sensitive patient information, so it should never operate using plain HTTP for protected data.

Use:

Patient/Doctor

|

| HTTPS

↓

TLS 1.3

|

↓

Telemedicine Server

Security provided by TLS

TLS provides:

- Confidentiality

- Integrity

- Server authentication

Instead of:

HTTP

Patient information → readable over network

use:

HTTPS

Patient information → encrypted

Example

Doctor

|

| HTTPS / TLS 1.3

|

↓

Telemedicine Portal

|

↓

Hospital Application Server

Important security measures

- TLS 1.3

- Valid digital certificates

- Strong authentication

- Role-based access control

- Session timeout

- Audit logs

- Secure password handling

- Encryption of sensitive information at rest

4. DICOM / Medical Imaging

The Diagnostic & Imaging Block produces large medical imaging files such as:

- MRI

- CT

- X-ray

- Ultrasound

These can be transferred to the hospital's PACS.

MRI / CT

|

↓

Imaging workstation

|

↓

Secure DICOM transfer

|

↓

PACS Server

|

↓

Authorised Doctor

Existing problem

Large DICOM files can consume significant bandwidth:

Large DICOM transfer

↓

High bandwidth usage

↓

Network congestion

↓

Telemedicine delay

Therefore, DICOM traffic should be classified appropriately in the QoS design.

It should have high priority, but emergency/real-time traffic should normally receive higher priority.

5. Secure File Transfer

Plain FTP should not be used for sensitive medical information because traditional FTP can transmit credentials and data without adequate encryption.

Instead use:

- SFTP

- HTTPS

- DICOM over a protected connection/TLS where supported

Example:

Radiology

|

| Encrypted transfer

↓

PACS

FTP vs SFTP

| Feature | FTP | SFTP |
| --- | --- | --- |
| Encryption | No by default | Yes |
| Credentials protected | No by default | Yes |
| Suitable for patient data | ❌ | ✅ |
| Secure transfer | Poor | Stronger |

Demonstration

You can demonstrate:

FTP

↓

Unencrypted credentials/data

SFTP

↓

Encrypted communication

Then explain why secure transfer is required for patient information.

6. SMTP for Critical Alerts

The hospital needs an email alert system for events such as:

- patient monitoring alerts

- device failures

- server failures

- network failures

- system warnings

Architecture:

Patient Monitor

↓

Monitoring Server

↓

Alert Engine

↓

SMTP Server

↓

Authorised Staff

Example:

Abnormal patient reading

↓

Alert server

↓

SMTP/TLS

↓

Doctor/Nurse

Existing problem

The problem statement mentions email alert failures.

Therefore, your design should include:

- authenticated SMTP

- TLS

- retry mechanism

- monitoring of mail-server availability

- alert logging

- backup notification mechanism where appropriate

A critical clinical alert should not depend solely on email; the system should have an appropriate clinical escalation mechanism.

7. SNMP Monitoring

SNMP can be used to monitor network infrastructure and supported medical-device gateways.

Use SNMPv3 rather than insecure legacy configurations where possible.

Medical IoT Gateway

|

| SNMPv3

↓

Monitoring Server

|

↓

Dashboard / Alert

Monitor:

- CPU utilisation

- memory

- interface status

- packet errors

- bandwidth utilisation

- device availability

Example:

Switch Port

↓

Errors increasing

↓

SNMP monitoring

↓

Administrator alert

This allows administrators to identify network/device problems before they become major failures.

8. Security Architecture

The hospital should use defence in depth.

INTERNET

|

[Firewall]

|

[VPN Gateway]

|

Core Network

|

+---------------+---------------+

|               |               |

ICU VLAN        IoT VLAN        Admin VLAN

|               |               |

ACL             ACL             ACL

Main security controls

| Security mechanism | Purpose |
| --- | --- |
| Firewall | Protect network perimeter |
| VLANs | Isolate departments |
| ACLs | Control communication |
| WPA3-Enterprise | Secure wireless |
| 802.1X | User/device authentication |
| TLS 1.3 | Encrypt applications |
| VPN | Secure remote access |
| SNMPv3 | Secure monitoring |
| RBAC | Restrict application access |
| Logging | Detect suspicious activity |
| Backups | Disaster/ransomware recovery |

9. Medical IoT Security

IoT devices are particularly important because there may be hundreds of:

- patient monitors

- infusion pumps

- wearable sensors

- gateways

Do not place them on the normal user network.

Use a dedicated:

MEDICAL-IOT VLAN

10.10.20.0/24

Example:

Patient Monitor

|

↓

Medical IoT VLAN

|

↓

IoT Gateway

|

↓

Monitoring Server

The ACL should allow only required communication.

For example:

Medical IoT → Monitoring Server       ALLOW

Medical IoT → DNS                     ALLOW

Medical IoT → Internet                DENY

Medical IoT → Admin PCs               DENY

Guest → Medical IoT                    DENY

This reduces the impact of a compromised IoT device.

10. Patient Data Privacy

Hospital systems contain highly sensitive information.

The design should protect:

Confidentiality

Only authorised users should access patient information.

Integrity

Patient records must not be modified without authorisation.

Availability

Critical information must remain available when doctors need it.

This is the:

CIA Triad

| Principle | Example |
| --- | --- |
| Confidentiality | TLS, ACLs, encryption |
| Integrity | TLS, authentication, logging |
| Availability | Redundant servers, links, backups |

11. HIPAA-like Considerations

Even though the assignment says HIPAA-like considerations, you should present these as general healthcare privacy/security principles rather than claiming that the simulated network itself is legally HIPAA compliant.

Important controls include:

- Minimum necessary access

- User authentication

- Role-based permissions

- Audit logging

- Encryption

- Secure transmission

- Backup and disaster recovery

- Incident response

- Access monitoring

Example:

Doctor

↓

Authenticated

↓

Role = Doctor

↓

Patient records

↓

Access allowed

But:

Guest user

↓

Not authorised

↓

Patient records

↓

ACCESS DENIED

12. Application Security Testing

Student D can demonstrate simple security tests.

Test 1 — Guest isolation

Try:

Guest PC → ICU Server

Expected:

ACCESS DENIED

Test 2 — Admin access

Admin PC → Admin Server

Expected:

ACCESS ALLOWED

Test 3 — IoT isolation

Medical IoT → Admin PC

Expected:

ACCESS DENIED

Test 4 — HTTPS

Client → HTTPS → Telemedicine Server

Expected:

Encrypted traffic

Test 5 — SNMP

Verify that the monitoring server can receive information from authorised devices.

13. Energy Efficiency

Wi-Fi 6 provides Target Wake Time (TWT) for compatible devices.

This is particularly useful for battery-powered medical sensors.

Without TWT:

Sensor

↓

Wake

↓

Listen

↓

Transmit

↓

Listen again

↓

Sleep

With TWT:

Sensor

↓

Sleep

↓

Scheduled wake-up

↓

Transmit

↓

Sleep

This can reduce unnecessary radio activity.

Benefits

- Longer sensor battery life

- Lower energy consumption

- Reduced wireless contention

- Better IoT scalability

14. Patient Safety

Network reliability directly affects patient safety.

For example:

Patient Monitor

↓

Medical IoT Network

↓

Monitoring Server

↓

Clinical Staff

If the network fails:

Network failure

↓

Monitoring interruption

↓

Delayed clinical information

↓

Potential patient risk

Therefore the network should provide:

- redundancy

- QoS

- reliable routing

- secure communication

- monitoring

- rapid fault detection

Critical network traffic should receive priority over guest and non-critical traffic.

15. Cost Analysis

The proposed solution has a higher initial cost because it requires:

- managed switches

- fibre backbone

- redundant equipment

- Wi-Fi 6 access points

- firewall

- monitoring

- secure servers

However:

Higher initial investment

↓

Better reliability

↓

Less downtime

↓

Lower operational risk

↓

Better patient service

The design is therefore justified for a hospital where network failure can have consequences beyond ordinary business disruption.

16. Sustainability

The network supports sustainability through:

Wi-Fi 6 TWT

Reduces battery consumption of sensors.

Telemedicine

Patients in remote locations may avoid unnecessary travel.

Remote patient

↓

Telemedicine

↓

Hospital specialist

This can improve accessibility and potentially reduce travel-related environmental impact.

Fibre

Fibre provides high bandwidth and can support large volumes of data efficiently over the backbone.

17. SDG Alignment

SDG 3 — Good Health and Well-being

The network supports:

- continuous monitoring

- telemedicine

- faster access to specialists

- reliable emergency services

Therefore:

Reliable network→better healthcare access

SDG 9 — Industry, Innovation and Infrastructure

The project uses modern infrastructure:

- Fibre

- Wi-Fi 6

- IoT

- SDN

- secure cloud/web technologies

- modern transport protocols

This represents resilient and innovative digital infrastructure.

SDG 11 — Sustainable Cities and Communities

Telemedicine can improve healthcare access for people who live far from specialist hospitals.

Remote area

|

| Internet

↓

Telemedicine

↓

Hospital specialist

19. Student D Overall Conclusion

Student D Conclusion:
The application layer of the proposed hospital network provides secure and reliable services for telemedicine, medical imaging, administration and monitoring. A hierarchical and caching DNS architecture reduces name-resolution delays, while HTTPS with TLS 1.3 protects telemedicine communication. DICOM and other medical files should be transferred using secure mechanisms rather than plaintext FTP. SMTP with authentication and encryption can support reliable system alerts, while SNMPv3 enables secure network and device monitoring. Network segmentation, ACLs, firewalls, authentication and encryption protect medical IoT devices and patient information. The proposed design also improves energy efficiency through Wi-Fi 6 Target Wake Time and supports telemedicine accessibility. Overall, the solution contributes to patient safety, sustainable infrastructure and the objectives of SDG 3, SDG 9 and SDG 11.